# COA Concepts — Simplest Possible Understanding

# 1. Binary Number System (Base-2)

## Idea

Computers understand only 0 and 1.

Like:

- Decimal → uses 0–9

- Binary → uses only 0 and 1

## Place Values

Binary place values are powers of 2.

| Binary | Value |
| --- | --- |
| 1 | 1 |
| 10 | 2 |
| 100 | 4 |
| 1000 | 8 |

Example:

### Convert (1011_2) to decimal

[ 1×2^3 + 0×2^2 + 1×2^1 + 1×2^0 ]

[ =8+0+2+1=11 ]

So:

[ 1011_2 = 11_{10} ]

## Memory Trick

Binary = powers of 2.

# 2. Hexadecimal System (Base-16)

Uses: [ 0-9, A,B,C,D,E,F ]

Where:

- A = 10

- B = 11

- …

- F = 15

## Why used?

Binary numbers become very long. Hex makes them short.

Example:

[ 1111_2 = F_{16} ]

## Easy Conversion

### Binary → Hex

Group binary into 4 bits.

Example:

[ 1010 1111 ]

- 1010 = A

- 1111 = F

Answer:

[ AF_{16} ]

## Memory Trick

1 Hex digit = 4 Binary bits

# 3. Signed Representation in Binary

Used to store positive and negative numbers.

## (a) Sign-Magnitude

- Leftmost bit = sign

- 0 → positive

- 1 → negative

Example:

| Binary | Meaning |
| --- | --- |
| 0101 | +5 |
| 1101 | -5 |

## (b) 1’s Complement

Invert all bits.

Example: [ 0101  ]

## (c) 2’s Complement (MOST IMPORTANT)

### Steps

- Invert bits

- Add 1

Example: Find -5

5: [ 0101 ]

Invert: [ 1010 ]

Add 1: [ 1011 ]

So: [ -5 = 1011 ]

## Why important?

Computers mostly use 2’s complement.

## Memory Trick

2’s complement = invert + 1

# 4. Floating Point Representation

Used for very large/small decimal numbers.

Example: [ 12345 = 1.2345 × 10^4 ]

Computer stores:

- Sign

- Exponent

- Mantissa

## Simple Analogy

Scientific notation in calculator.

## Formula

[ Number = Mantissa × Base^{Exponent} ]

# 5. Flip-Flop

## Idea

A flip-flop stores 1 bit of data.

Like a tiny memory switch.

States:

- 0

- 1

# Types of Flip-Flops

| Type | Meaning |
| --- | --- |
| SR | Set Reset |
| JK | Improved SR |
| D | Data |
| T | Toggle |

# D Flip-Flop (Easy One)

## Rule

Output follows input on clock pulse.

| D | Q(next) |
| --- | --- |
| 0 | 0 |
| 1 | 1 |

## Analogy

Like taking a photo when clock clicks.

# Example Circuit

D Flip-Flop:

- Input = D

- Clock pulse controls storing

- Output = Q

Used in:

- Registers

- Counters

- Memory

# 6. Pulse Triggering

Clock pulse tells circuit WHEN to work.

## Types

### Level Triggering

Works while pulse stays HIGH/LOW.

### Edge Triggering

Works only at transition.

- Rising edge ↑

- Falling edge ↓

## Memory Trick

Edge triggering = action only at edge.

# 7. Shift Registers

A group of flip-flops used to shift data.

## Data movement

- Left shift

- Right shift

Example: [ 1010  ]

## Uses

- Data transfer

- Multiplication/division

- Serial communication

## Types

| Type | Meaning |
| --- | --- |
| SISO | Serial In Serial Out |
| SIPO | Serial In Parallel Out |
| PISO | Parallel In Serial Out |
| PIPO | Parallel In Parallel Out |

# 8. Counter

Counter counts clock pulses.

Example: 0 → 1 → 2 → 3 …

Built using flip-flops.

## Types

### Asynchronous Counter

Slow (ripple effect)

### Synchronous Counter

Fast (same clock)

## Uses

- Digital clocks

- Timers

- Frequency counting

# 9. Pipeline Execution

## Idea

Execute many instructions simultaneously in stages.

## Real-Life Analogy

Laundry: 1. Wash 2. Dry 3. Fold

While one cloth dries, another washes.

CPU also overlaps tasks.

# Pipeline Stages

| Stage | Work |
| --- | --- |
| IF | Fetch instruction |
| ID | Decode |
| EX | Execute |
| MEM | Memory access |
| WB | Write back |

## Benefit

Increases speed.

# 10. Instruction Pipeline

Pipeline specifically for instruction execution.

Many instructions processed together.

Example:

| Clock | Inst1 | Inst2 | Inst3 |
| --- | --- | --- | --- |
| 1 | Fetch |  |  |
| 2 | Decode | Fetch |  |
| 3 | Execute | Decode | Fetch |

# 11. Pipeline Hazards

Problems that stop smooth pipelining.

3 Types: 1. Data Hazard 2. Control Hazard 3. Structural Hazard

# 12. Data Hazard

Occurs when instruction needs data not ready yet.

Example:

A = B + C
D = A + E

Second instruction waits for A.

## Solution

- Forwarding

- Stall

## Memory Trick

Data dependency problem.

# 13. Control Hazard

Occurs due to branch/jump instructions.

CPU doesn’t know next instruction.

Example:

if (x>0)

CPU waits for decision.

## Solution

- Branch prediction

- Pipeline flush

## Memory Trick

Decision-making problem.

# 14. Cache Mapping

Cache = small fast memory.

Mapping decides where data goes in cache.

# Types

## (a) Direct Mapping

One memory block → one cache line

Simple but collisions occur.

## (b) Associative Mapping

Any block anywhere.

Fast but expensive.

## (c) Set Associative Mapping

Combination of both.

Most commonly used.

# Super Easy Revision Table

| Topic | One-Line Idea |
| --- | --- |
| Binary | Base-2 numbers |
| Hexadecimal | Short form of binary |
| Signed Binary | Store negative numbers |
| Floating Point | Scientific notation in computer |
| Flip-Flop | Stores 1 bit |
| Pulse Triggering | Clock controls timing |
| Shift Register | Shifts data |
| Counter | Counts pulses |
| Pipeline | Parallel instruction execution |
| Instruction Pipeline | Instruction stages overlap |
| Pipeline Hazard | Pipeline problem |
| Data Hazard | Data not ready |
| Control Hazard | Branch confusion |
| Cache Mapping | How cache stores data |